Pumping test field sheet: borehole KTL-01

Figure 1. Sheet code GWT-FK/1|pumping|Rokel 2026|KTL-01. Scan it, or type it in, to match this sheet to its project and borehole.

| Community | Kuntolo | Date |  |
| --- | --- | --- | --- |
| Client | Living Water International | Length of each step (min) |  |
| Test conducted by | WiNGiN | Start time |  |
| Borehole Ref. No. | KTL-01 | Depth of Borehole (m) |  |
| Static water level (m) |  | Pump setting (m) |  |
| Test type (step or constant) | constant | District | Port Loko |
| GPS Coordinate East |  | GPS Coordinate North |  |
| UTM Zone (28N or 29N) |  | Elevation (m) |  |

Casing storage: if the transmissivity is as low as 1 m2/day, the water levels of the first 307 minutes may show the 5 inch casing, with its 1.25 inch riser, emptying rather than the aquifer (31 minutes at 10 m2/day).

Constant discharge test: do not stop the pump before 307 minutes, the longer of that time and the shortest constant test the analysis accepts, 240 minutes.

Step test: run every step for at least 60 minutes, and the first for at least 307 minutes, since the drawdown is fitted on the first step.

A shorter test is safe only once the measured drawdown shows a shorter casing-storage time. The pumping test co-pilot or the office works that out from the discharge and the drawdown.

Read the water level at each minute in the Time column, counted from the moment the pump starts. After 120 minutes, read every 30 minutes.

When the pump stops, start the clock again from zero and read the recovery at the same minutes.

Record the discharge of every step. Time a bucket of known volume three times; the rate in m3/h is 3.6 times the litres divided by the mean seconds. A step with no discharge is written as Not measured, with the reason.

Table 1. Discharge per step (m3/h)

| Step 1 Q | Step 2 Q | Step 3 Q | Step 4 Q |
| --- | --- | --- | --- |

Table 2. Bucket timings

| Step | Bucket (L) | Timing 1 (s) | Timing 2 (s) | Timing 3 (s) | Q (m3/h) |
| --- | --- | --- | --- | --- | --- |
| Step 1 |  |  |  |  |  |
| Step 2 |  |  |  |  |  |
| Step 3 |  |  |  |  |  |
| Step 4 |  |  |  |  |  |

The tables below have the columns of the standard pumping test workbook, so a filled sheet is typed into it cell for cell.

Table 3. Constant discharge 0-60 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 0.5 |  |  |
| 1 |  |  |
| 1.5 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 8 |  |  |
| 10 |  |  |
| 12 |  |  |
| 15 |  |  |
| 20 |  |  |
| 25 |  |  |
| 30 |  |  |
| 40 |  |  |
| 50 |  |  |
| 60 |  |  |

Table 4. Constant discharge 61-120 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 75 |  |  |
| 90 |  |  |
| 120 |  |  |

Table 5. Constant discharge 121-180 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 150 |  |  |
| 180 |  |  |

Table 6. Constant discharge 181-330 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 210 |  |  |
| 240 |  |  |
| 270 |  |  |
| 300 |  |  |
| 330 |  |  |

Table 7. Recovery

| Time (min) | Water Level (m) | Recovery (m) |
| --- | --- | --- |
| 0.5 |  |  |
| 1 |  |  |
| 1.5 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 8 |  |  |
| 10 |  |  |
| 12 |  |  |
| 15 |  |  |
| 20 |  |  |
| 25 |  |  |
| 30 |  |  |
| 40 |  |  |
| 50 |  |  |
| 60 |  |  |
| 75 |  |  |
| 90 |  |  |
| 120 |  |  |
| 150 |  |  |
| 180 |  |  |
| 210 |  |  |
| 240 |  |  |
| 270 |  |  |
| 300 |  |  |
| 330 |  |  |

Pumping test field sheet: borehole KTL|02 %x

Figure 2. Sheet code GWT-FK/1|pumping|Rokel 2026|KTL%7C02 %25x. Scan it, or type it in, to match this sheet to its project and borehole.

| Community | Kuntolo | Date |  |
| --- | --- | --- | --- |
| Client | Living Water International | Length of each step (min) |  |
| Test conducted by | WiNGiN | Start time |  |
| Borehole Ref. No. | KTL¦02 %x | Depth of Borehole (m) |  |
| Static water level (m) |  | Pump setting (m) |  |
| Test type (step or constant) | constant | District | Port Loko |
| GPS Coordinate East |  | GPS Coordinate North |  |
| UTM Zone (28N or 29N) |  | Elevation (m) |  |

Casing storage: if the transmissivity is as low as 1 m2/day, the water levels of the first 307 minutes may show the 5 inch casing, with its 1.25 inch riser, emptying rather than the aquifer (31 minutes at 10 m2/day).

Constant discharge test: do not stop the pump before 307 minutes, the longer of that time and the shortest constant test the analysis accepts, 240 minutes.

Step test: run every step for at least 60 minutes, and the first for at least 307 minutes, since the drawdown is fitted on the first step.

A shorter test is safe only once the measured drawdown shows a shorter casing-storage time. The pumping test co-pilot or the office works that out from the discharge and the drawdown.

Read the water level at each minute in the Time column, counted from the moment the pump starts. After 120 minutes, read every 30 minutes.

When the pump stops, start the clock again from zero and read the recovery at the same minutes.

Record the discharge of every step. Time a bucket of known volume three times; the rate in m3/h is 3.6 times the litres divided by the mean seconds. A step with no discharge is written as Not measured, with the reason.

Table 8. Discharge per step (m3/h)

| Step 1 Q | Step 2 Q | Step 3 Q | Step 4 Q |
| --- | --- | --- | --- |

Table 9. Bucket timings

| Step | Bucket (L) | Timing 1 (s) | Timing 2 (s) | Timing 3 (s) | Q (m3/h) |
| --- | --- | --- | --- | --- | --- |
| Step 1 |  |  |  |  |  |
| Step 2 |  |  |  |  |  |
| Step 3 |  |  |  |  |  |
| Step 4 |  |  |  |  |  |

The tables below have the columns of the standard pumping test workbook, so a filled sheet is typed into it cell for cell.

Table 10. Constant discharge 0-60 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 0.5 |  |  |
| 1 |  |  |
| 1.5 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 8 |  |  |
| 10 |  |  |
| 12 |  |  |
| 15 |  |  |
| 20 |  |  |
| 25 |  |  |
| 30 |  |  |
| 40 |  |  |
| 50 |  |  |
| 60 |  |  |

Table 11. Constant discharge 61-120 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 75 |  |  |
| 90 |  |  |
| 120 |  |  |

Table 12. Constant discharge 121-180 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 150 |  |  |
| 180 |  |  |

Table 13. Constant discharge 181-330 min

| Time (min) | Water Level (m) | Drawdown (m) |
| --- | --- | --- |
| 210 |  |  |
| 240 |  |  |
| 270 |  |  |
| 300 |  |  |
| 330 |  |  |

Table 14. Recovery

| Time (min) | Water Level (m) | Recovery (m) |
| --- | --- | --- |
| 0.5 |  |  |
| 1 |  |  |
| 1.5 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 8 |  |  |
| 10 |  |  |
| 12 |  |  |
| 15 |  |  |
| 20 |  |  |
| 25 |  |  |
| 30 |  |  |
| 40 |  |  |
| 50 |  |  |
| 60 |  |  |
| 75 |  |  |
| 90 |  |  |
| 120 |  |  |
| 150 |  |  |
| 180 |  |  |
| 210 |  |  |
| 240 |  |  |
| 270 |  |  |
| 300 |  |  |
| 330 |  |  |

Quick card: pumping test reading schedule

Read the water level at these minutes after the pump starts, then every 30 minutes.

When the pump stops, start the clock again from zero and read the recovery at the same minutes.

Table 15. Reading minutes

| 0.5 | 1 | 1.5 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| 6 | 8 | 10 | 12 | 15 | 20 | 25 |
| 30 | 40 | 50 | 60 | 75 | 90 | 120 |

Casing storage: if the transmissivity is as low as 1 m2/day, the water levels of the first 307 minutes may show the 5 inch casing, with its 1.25 inch riser, emptying rather than the aquifer (31 minutes at 10 m2/day).

Constant discharge test: do not stop the pump before 307 minutes, the longer of that time and the shortest constant test the analysis accepts, 240 minutes.

Step test: run every step for at least 60 minutes, and the first for at least 307 minutes, since the drawdown is fitted on the first step.

A shorter test is safe only once the measured drawdown shows a shorter casing-storage time. The pumping test co-pilot or the office works that out from the discharge and the drawdown.

Quick card: VES Schlumberger spacings

A sounding reaches a depth of about 0.5 times its largest AB/2. Peg down the table until the depth reached is at least the target depth, and stop there.

AB is always at least 5 times MN, and MN is widened before AB passes 20 times it. Where MN changes, read the same AB/2 again with the new MN, and keep both readings.

Table 16. Schlumberger spacings

| No. | AB/2 (m) | MN (m) | Depth reached (m) |
| --- | --- | --- | --- |
| 1 | 1 | 0.4 | 0.5 |
| 2 | 1.5 | 0.4 | 0.75 |
| 3 | 2 | 0.4 | 1 |
| 4 | 3 | 0.4 | 1.5 |
| 5 | 4 | 0.4 | 2 |
| 6 | 4 | 1 | 2 |
| 7 | 5 | 1 | 2.5 |
| 8 | 6 | 1 | 3 |
| 9 | 8 | 1 | 4 |
| 10 | 10 | 1 | 5 |
| 11 | 10 | 2 | 5 |
| 12 | 12 | 2 | 6 |
| 13 | 15 | 2 | 7.5 |
| 14 | 20 | 2 | 10 |
| 15 | 20 | 5 | 10 |
| 16 | 25 | 5 | 12.5 |
| 17 | 30 | 5 | 15 |
| 18 | 40 | 5 | 20 |
| 19 | 50 | 5 | 25 |
| 20 | 50 | 20 | 25 |
| 21 | 60 | 20 | 30 |
| 22 | 80 | 20 | 40 |
| 23 | 100 | 20 | 50 |
| 24 | 120 | 20 | 60 |
| 25 | 150 | 20 | 75 |
| 26 | 200 | 20 | 100 |
| 27 | 200 | 40 | 100 |
| 28 | 250 | 40 | 125 |
| 29 | 300 | 40 | 150 |
| 30 | 400 | 40 | 200 |
| 31 | 400 | 100 | 200 |
| 32 | 500 | 100 | 250 |

Quick card: disinfecting the borehole with chlorine

Shock dose of 20 mg of chlorine per litre: add 1 litre of 0.2 percent chlorine solution for every 100 litres of water standing in the well, and do not pump for at least 4 hours.

A 0.2 percent solution holds 2 g of chlorine per litre: dissolve about 3.1 g of 65 percent HTH powder in each litre of water.

Water in the well, in litres, is 0.785 times the square of the casing inner diameter in mm, times the water column in m, divided by 1,000.

Table 17. Litres of 0.2 percent solution (L) and grams of 65 percent HTH (g), by water column and casing inner diameter.

| Water column (m) | 100 mm: L | 100 mm: g | 125 mm: L | 125 mm: g | 150 mm: L | 150 mm: g |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 0.8 | 2 | 1.2 | 4 | 1.8 | 5 |
| 20 | 1.6 | 5 | 2.5 | 8 | 3.5 | 11 |
| 30 | 2.4 | 7 | 3.7 | 11 | 5.3 | 16 |
| 40 | 3.1 | 10 | 4.9 | 15 | 7.1 | 22 |
| 50 | 3.9 | 12 | 6.1 | 19 | 8.8 | 27 |
| 60 | 4.7 | 14 | 7.4 | 23 | 10.6 | 33 |

Basis: the toolkit's supervision checklist (borehole disinfected with chlorine at 20 mg per litre, after Adekile, 2014) and its dose calculator, which follows WHO. Where the programme's own disinfection procedure sets a different dose, it takes precedence.

# References

Adekile, D. (2014). Supervising Water Well Drilling: a Guide for Supervisors. RWSN/Skat, St Gallen.

World Health Organization (2022). Guidelines for Drinking-water Quality, 4th edition incorporating the first and second addenda. Geneva: World Health Organization.